- Interfacing Arduino with pot and servo motor

#include <Servo.h>

Servo myServo;

int potPin = A0;

int servoPin = 9;

int potValue;

int angle;

void setup() {

myServo.attach(servoPin);

}

void loop() {

potValue = analogRead(potPin);      // Read potentiometer (0–1023)

angle = map(potValue, 0, 1023, 0, 180);  // Convert to angle (0–180)

myServo.write(angle);               // Move servo

delay(15);                          // Small delay for smooth movement

}

- Interfacing Arduino with stepper motor

#include <Stepper.h>

Const int apr = 40

Stepper motor(apr, 8, 9, 10, 11);   // steps, pins

void setup() {

motor.setSpeed(10);              // set speed

}

void loop() {

motor.step(apr);    // move forward

delay(1000);

motor.step(-apr);   // move backward

delay(1000);

}

Interfacing Arduino with analog sensor – control led basd on ldr

int ldrValue;

void setup() {

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void loop() {

ldrValue = analogRead(A0);

if (ldrValue < 500) {

digitalWrite(10, HIGH);   // LED 1 ON

digitalWrite(11, LOW);    // LED 2 OFF

} else {

digitalWrite(10, LOW);    // LED 1 OFF

digitalWrite(11, HIGH);   // LED 2 ON

}

delay(200);

}

6. Interfacing DHT22 Sensor with Arduino

#include <DHT.h>

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

DHT dht(13, DHT11);

void setup() {

lcd.begin(16, 2);

dht.begin();

}

void loop() {

float t = dht.readTemperature();

float h = dht.readHumidity();

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(t);

lcd.print("C");

lcd.setCursor(0, 1);

lcd.print("Hum: ");

lcd.print(h);

lcd.print("%");

delay(1000);}

4 Write a program to interface LED/Buzzer with Arduino and to turn ON LED for 1 second after every 2 seconds.

const int LED=10;

void setup()

{

pinMode(LED,OUTPUT);

}

void loop()

{

digitalWrite(LED,HIGH);

delay(1000);

digitalWrite(LED,LOW);

delay(2000);

}